1.

Writing a node

ROS 2 installed (e.g., Humble, Iron, Jazzy)

A ROS 2 workspace created

Create a Python package

Write node

import rclpy

from rclpy.node import Node

class MinimalNode(Node):

def __init__(self):

super().__init__('minimal_node')

self.get_logger().info('Hello from my first ROS2 node!')

def main(args=None):

rclpy.init(args=args)

node = MinimalNode()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

Edit setup.py to include the entry point:

'simple_node = my_first_pkg.simple_node:main'

Build the Package

Run the Node

Example of publisher

self.publisher_ = self.create_publisher(String, 'chatter', 10)

self.timer = self.create_timer(1.0, self.timer_callback)

Example of Subscriber

#type of message, topic name, callback function, QoS size

self.subscription = self.create_subscription(String, '/chatter', self.listener_callback,10)

self.subscription  # prevent unused variable warning

self.get_logger().info('Listener node has been started.')

build and source

run subscriber node

2.

Always use a new terminal for every command

ros2 launch turtlebot3_gazebo lab02.launch.py

run the command above to launch gazebo with maze and robot

ros2 launch turtlebot3_bringup rviz2.launch.py

visualize the robot and its sensors using the above launch file

Bump&Go controller:

Checkout the code! Ask questions if any!

3.

What is zenoh? -Zenoh is an emerging data communication protocol and middleware that’s gaining attention in robotics, IoT, and distributed systems — including integration with ROS

On the robot PC

Start zenoh for data transfer to own pc, launch robot in different terminals

ros2 run rmw_zenoh_cpp rmw_zenohd

ros2 launch turtlebot3_bringup robot.launch.py

on own PC run the following command to see what robot sees

ros2 launch turtlebot3_bringup rviz2.launch.py

for keyboard teleop

ros2 run turtlebot3_teleop teleop_keyboard

To connect to the robot checkout the guide for lab 3!